--------------------------------------------------------------------------------

--

--   AM2901 Benchmark

--

-- Source: AMD data book

--

-- VHDL Benchmark author Indraneel Ghosh

--                       University Of California, Irvine, CA 92717

--

-- Developed on Nov 1, 1991

--

--------------------------------------------------------------------------------

This is a behavioral or algorithmic model of the Am2901, in which it is

modeled as a single VHDL process, without any partitioning into smaller

blocks.

-----------------------------------------------------------------

VARIABLES, TYPES AND CONSTANTS :

File "am2901.doc" contains a general description of the chip and its ports.

This model uses the following types and variables:

*******************************************************

TYPES :

Memory ==> An array of MVL4-vectors ( analogous to bit-vectors).

Each MVL4-vector is of length 4.

*******************************************************

VARIABLES :

RAM ==> Used to represent the 4-bit by 16-word RAM. ( data type "Memory")

A ==> The RAM word addressed by Aadd is read into this variable. ( 4 bits)

B ==> The RAM word addressed by Badd is read into this variable. ( 4 bits)

Q ==> Used to represent the Q register. ( 4 bits)

RE ==> Input data for ALU, selected by ALU data selector. ( 4 bits)

S ==> Input data for ALU, selected by ALU data selector. ( 4 bits)

R_ext ==> Extended input data for ALU. This consists of "RE"

concatenated with a '0' bit at the MSB position.

This extended ALU input is used to facilitate

computation of carry bit "C4". ( 5 bits)

( R_ext := '0' & RE; )

S_ext ==> Extended input data for ALU. This consists of "S"

concatenated with a '0' bit at the MSB position.

This extended ALU input is used to facilitate

computation of carry bit "C4". ( 5 bits)

( R_ext := '0' & S; )

result ==> Extended output from ALU. ( 5 bits)

F ==> Normal ALU output, obtained from the "result" by leaving out its

MSB. ( 4 bits)

( F := result(3 downto 0);  )

temp_p ==> An intermediate variable, calculated from "result" and used

in the computation of propagate term "Pbar". ( 5 bits)

temp_g ==> An intermediate variable, calculated from "result" and used

in the computation of generate term "Gbar". ( 5 bits)

dout ==> An intermediate term used in the computation of data output "Y".

If OEbar is asserted, "dout" is fed to "Y". Else "Y" is

assigned HIGH-Z. ( 4 bits)

-----------------------------------------------------------------

GENERAL DESCRIPTION OF MODEL :

The VHDL process is trigerred by a falling clock edge.

The VHDL process has three case statements corresponding to :

## ALU operand selection

## ALU function selection

## ALU destination and data-output selection.

In the model, data is first read into "A" and "B" from the RAM words

addressed by Aadd and Badd. Then the ALU operands are selected. The ALU

does computation on these operands. After that, the destination selector

decides whether and how to write the ALU results to the RAM and Q register.

For further details, its best to look at the model which has detailed comments

and also the file "am2901.doc".