--------------------------------------------------------------------------------

--

--   DOCUMENTATION ON THE GREATEST COMMON DIVISIOR

--

-- Source : "Algorithmics" by Brassard and Bradley

--

-- author: Ted Lee

--         University Of California, Irvine, CA 92717

--         tedl@ics.uci.edu

--

--   Developed on Feb 19, 1992

--

--------------------------------------------------------------------------------

OVERVIEW

The gcd model described in this example consists of two 8-bit input ports,

one 8-bit output port, and a one-bit input port that enables the gcd model

when the value is low.

_______________________________________________________________________________

PORTS

It has the following ports :-

_______________________________________________________________________________

|  PORT    | TYPE | BIT   |                DESCRIPTION                        |

|	   |      | WIDTH |						      |

|==========|======|=======|===================================================|

| xi	   |  in  |  8    | Input x                                           |

|__________|______|_______|___________________________________________________|

| yi       |  in  |  8    | Input y                                           |

|__________|______|_______|___________________________________________________|

| ou       | out  |  8    | Output ou = gcd(xi, yi)                           |

|__________|______|_______|___________________________________________________|

| rst      |  in  |  1    | ou = gcd(xi, yi) only when rst = '0'              |

|__________|______|_______|___________________________________________________|

_______________________________________________________________________________

OPERATION

The gcd model can be enabled by setting rst='0'. The output ou will be

evaluated as gcd(xi, yi) based on the input values of xi and yi. If rst='1',

then the output will be evaluated to be "00000000".